AMZL UK & IE

Roadworthiness Guideline

# Overview

This Roadworthiness Guideline defines defects for 1.0 and 2.0 vehicles. It provides a ‘yes’ or ‘no’ answer to the question of ‘whether damage is acceptable?’. The defects in this Guideline are treated as either:

- ‘Sev 1’, and the vehicle is immediately grounded; or

- ‘Sev 2’ meaning the vehicle must be repaired within 14 days otherwise it becomes automatically grounded.

The defects below are Sev 1 unless specifically stated and ordered by commonality of defects.

As outlined in the Fleet Standards and Requirements (FSR), the Roadworthiness Guidelines specify the criteria for unacceptable damage that DSPs must adhere to when providing services for Amazon. These guidelines include visual evidence of damages observed in the fleet to assist in determining the acceptability of certain damages.

Damages could be inside or outside the vehicle.

Contents

Overview 	1

Defect Definition Overview 	3

Side Mirrors 	5

Lights and Light Covers 	8

Tires, Wheels and Rims 	10

Body and Doors – Holes and Attachments 	12

Body and Doors – Holes and Attachments Continued 	13

Body and Doors – Holes and Attachments Continued 	14

Body and Doors – Holes and Attachments Continued 	15

Body and Doors – Sharp edges 	15

Body and Doors – Fuel and Fluids 	16

Body and Doors – Wires 	18

Body and Doors – Netradyne 	19

Body and Doors – Amazon Prime decal 	20

License Plate 	21

Safety Accessories 	22

Vehicle Cleanliness 	22

Seatbelt 	23

Windshield 	24

Camera 	25

# Defect Definition Overview

| Damage Category Vehicle Safety Audit Requirements |  |
| --- | --- |
| Side Mirrors | Mirror is missing or cracked Indicator light or cover is cracked (by 30mm or more), have a hole, covered, missing, not working or wrong colour Casing is broken, missing, held on with tape/zip ties Side mirrors are loose, hanging, unsecured or held up with a zip-tie, tape, or similar Side mirrors cannot be adjusted |
| Lights and Light Covers | Light or light cover is cracked (by 50mm or more), has a hole, is covered or missing Headlight is not working Tail or brake light is not working Fog light is not working Hazard light or turn signal is not working or not amber More than 50% of a rear reflector is missing or reflective tape used |
| Tire, Wheel and Rim | Tire is flat, leaking air, bulging or swelling Tire has objects, cuts (more than 25mm), dents, or exposed ply or cord Tire has insufficient tread (Less than 1.6mm) Tire has low tread (1.6mm to 1.7mm) – Sev 2 Wheel or rim is damaged, cracked, or broken Wheel nuts are missing, or loose |
| Body and Doors | Doors cannot open, close, or lock correctly Fuel cap/charging port is missing, broken or not lockable Hole in the bodywork – Sev 2 Items attached to the body of the vehicle (for example: bumper, front or reversing sensors, cargo step) are missing, damaged, loose, unsecure, hanging, or held with a zip-tie, tape, or similar Leaking fluid Loose, hanging, faulty, or frayed wire Sharp edge likely to cause injury or harm when contacted or grazed Where Netradyne is installed, external privacy sticker is missing or is not clearly visible from 6m away (UK 2.0 fleet only) Amazon Prime decal is damaged, missing, or excessively dirty or any graffiti on an Amazon branded vehicle – Sev 2 |

| Damage Category | Vehicle Safety Audit Requirements |
| --- | --- |
| Windshield | Driver’s line of sight is obscured through dirt, stickers or devices (e.g. device cradle) Windshield has crack or chips more than 10mm in driver’s line of sight or 40mm outside the driver’s line of sight Wipers are missing, damaged, or not working Windshield washer system is not working and/or wiper fluid reservoir is empty |
| License Plate | License plate is damaged, missing or obscured |
| Safety Accessories | Delivery device cradle is damaged, missing, or is mounted with a tape, zip-tie or similar |
| Camera/Monitor | Back-up camera is loose, hanging, missing, or mounted with a tape, zip-tie, or similar |
| Seatbelt | Seatbelt or seatbelt buckle is missing, torn, frayed, damaged or not working |
| Vehicle Cleanliness | Exterior of vehicle is visibly dirty when viewed from 6m away – Sev 2 Interior of vehicle has excessive grime, dust, or trash present Unapproved advertisement on a DSP 2.0 vehicle – Sev 2 |

# Side Mirrors

Example

Groundable

Damage

Definition

•

Mirror or glass is missing or cracked.

•

Indicator light or cover is cracked (by 30mm or more), has a hole, covered, missing, not working or

wrong colour.

•

Casing is broken, missing, held on with tape

/

zip ties

.

•

Side mirrors are loose, hanging, unsecured, or held up with a zip

-

tie, tape, or similar.

•

Side mirrors cannot be adjusted.

Example

Groundable

Damage

Definition

•

Mirror or glass is missing or cracked.

•

Indicator light or cover is cracked (by 30mm or more), has a hole, covered, missing, not working or

wrong colour.

•

Casing is broken, missing, held on with tape

/

zip ties

.

•

Side mirrors are loose, hanging, unsecured, or held up with a zip

-

tie, tape, or similar.

•

Side mirrors cannot be adjusted.

Plastic casing is broken

Plastic casing is missing. 	with sharp edges. 	Mirror is cracked. 	Mirror is missing.

Ground. 	Ground. 	Ground. 	Ground.

## Example

Side mirrors held on with tape or covered with a bag. Neither of these provide appropriate support or protection. Ground.

# Side Mirrors Continued

## Example

Side mirrors cannot be adjusted due to obstructions and tape. Ground.

## Example

Indicator light or cover is cracked (by 30mm or more), has a hole, covered, missing, or not working. Ground.

Fleet Compliance Tip Fleet Compliance Tip

The following damage is not deemed sufficient to impact safety. Do not ground for the following damage.

## Example

Plastic casing is damaged, but no sharp edges or exposed electrical wires. Do not ground.

# Side Mirrors Continued

Fleet Compliance

Tip

-

Side

Indicator

Lights

Side

(

Repeaters)

Ensure that each side repeater is placed in a manner where at least half of its visible surface is

observable from any point within the below specified angles.

It is important we check the side repeater is not:

•

Missing

•

Cracked

•

Broken

If any of the above, the vehicle must be grounded and the defect fixed. To note, the vehicle must have both

side indicators (side repeater(s)). These can be placed on either the side wing mirror or side of body (as

shown in images below).

Fleet Compliance

Tip

-

Side

Indicator

Lights

Side

(

Repeaters)

Ensure that each side repeater is placed in a manner where at least half of its visible surface is

observable from any point within the below specified angles.

It is important we check the side repeater is not:

•

Missing

•

Cracked

•

Broken

If any of the above, the vehicle must be grounded and the defect fixed. To note, the vehicle must have both

side indicators (side repeater(s)). These can be placed on either the side wing mirror or side of body (as

shown in images below).

Example

The side indicator must be amber. White, blue or any other colour is not permitted and must be grounded.

Amber indicator light.

Do not ground.

Here are some examples of side indicators:

White indicator light. Ground.

# Lights and Light Covers

Groundable

Damage

Definition

•

Light or light cover is cracked (by 50mm or more), has a hole, covered or missing

.

•

Head light

is

not

working.

•

Tail

or brake

light

is

not working.

•

Fog

light

is

not

working.

•

Hazard light or turn signal is not working or not amber

.

•

License

plate

light

is

not

working.

•

More than 50% of a r

ear

reflector

is

missing

or reflective tape used

.

Groundable

Damage

Definition

•

Light or light cover is cracked (by 50mm or more), has a hole, covered or missing

.

•

Head light

is

not

working.

•

Tail

or brake

light

is

not working.

•

Fog

light

is

not

working.

•

Hazard light or turn signal is not working or not amber

.

•

License

plate

light

is

not

working.

•

More than 50% of a r

ear

reflector

is

missing

or reflective tape used

.

## Example

Hole in light. Ground. 	Light is covered with tape. Ground.

Example

Example

Cracked lights more than 50mm. Ground. 	 	Fog light is not working/ and or is missing. 	 Ground.

# Lights and Light Covers Continued

## Example

Two rear reflectors are required. Neither can have more than 50% missing.

Reflective tape is not an acceptable substitute for a rear reflector.

Some vehicles have the rear

reflector incorporated within

the rear lights. This is

acceptable and should not be

grounded.

Some vehicles have the rear

reflector incorporated within

the rear lights. This is

acceptable and should not be

grounded.

## Example

More than 50% of the rear reflector is missing. Ground. 	Rear reflector is missing. Ground.

## Example

Reflective tape used as a rear reflector. Tape is not an acceptable substitute. Ground.

# Tires, Wheels and Rims

Groundable

Damage

Definition

•

Tire is flat, leaking air, bulging, or swelling.

•

Tire has objects, cuts (more than 25mm), dents, or exposed ply or cord.

•

Tire has insufficient tread (Less than 1.6mm).

•

Tire has low tread (1.6mm to 1.7mm)

–

Sev 2.

•

Wheel

rim,

or

mounting

equipment

is

damaged,

cracked,

or

broken.

•

Wheel

nuts

are

missing,

or

loose.

Groundable

Damage

Definition

•

Tire is flat, leaking air, bulging, or swelling.

•

Tire has objects, cuts (more than 25mm), dents, or exposed ply or cord.

•

Tire has insufficient tread (Less than 1.6mm).

•

Tire has low tread (1.6mm to 1.7mm)

–

Sev 2.

•

Wheel

rim,

or

mounting

equipment

is

damaged,

cracked,

or

broken.

•

Wheel

nuts

are

missing,

or

loose.

## Example

Missing wheel nut. Ground.             Wheel rim is damaged. Ground.

## Example

Cut in tire more than 25mm. Exposed cord.                      Nail in tire.

Ground.                                Ground.                               Ground.

# Tires, Wheels and Rims Continued

The primary grooves of the tread pattern must be at least 1.6mm deep:

Within the central three-quarters of the breadth of tread. Around the entire outer circumference of the tire.

## Example

Tires with insufficient tread (below 1.6mm). Ground.

Compliance Tip Compliance Tip

You can manually check if there is insufficient tread with a tire depth gauge or by checking the tread indicators which are molded into the tread grooves in regular intervals. If the tire is flush with the bars, the tire is below 1.6mm. Make sure to check all ¾ tread grooves as tires can wear unevenly.

Example

Example

# Body and Doors – Holes and Attachments

Groundable

Damage

Definition

•

Hole in bodywork

-

Sev 2.

•

Items attached to the body of the vehicle (for example: bumper,

front / rear

sensors, cargo steps) are

missing, damaged, loose, unsecure, hanging, or held with a zip

-

tie, tape, or similar.

•

Doors cannot open, close, or lock

correctly.

•

Loose or hanging objects or frayed wires.

Groundable

Damage

Definition

•

Hole in bodywork

-

Sev 2.

•

Items attached to the body of the vehicle (for example: bumper,

front / rear

sensors, cargo steps) are

missing, damaged, loose, unsecure, hanging, or held with a zip

-

tie, tape, or similar.

•

Doors cannot open, close, or lock

correctly.

•

Loose or hanging objects or frayed wires.

## Example

Holes in the bodywork – these allow water in and can damage parcels.

## Example

Tape does not provide appropriate support or protection. Bodywork held on with tape. Ground.

## Example

Doors cannot open, close, or lock correctly. Ground.

Compliance Tip Compliance Tip

Slips, trips and falls are common injury causes for DAs. Where there is safety equipment e.g. cargo steps and sensors, these must be safe to use.

Example

s

of cargo steps

:

Example

s

of cargo steps

:

Compliance Tip Compliance Tip

Sensors are crucial to the Delivery Associates’ safety when out on road, ensure you check the condition of these as per the checklist.

Example

s of sensors:

Example

s of sensors:

Compliance Tip

Items that are missing, damaged, loose, unsecure, hanging, or held with a zip-tie, tape, or similar - if the body part is likely to fall off, cause harm or injury when contacted or grazed, this is illegal - it must be grounded.

Examples:

Ground these vehicles

Items attached to the body of the vehicle are missing, damaged, loose, unsecure, hanging, or held with a ziptie, tape, or similar. If the body part is likely to fall off or cause harm it must be grounded.

Do not ground these vehicles

Whilst there is cosmetic damage (missing trim) there are no sharp edges, and the doors can open and close as normal.

Whilst the bumper has been

secured with a screw it is not

loose, unsecure, or hanging.

Therefore, it is unlikely to

fall off.

Whilst the bumper has been

secured with a screw it is not

loose, unsecure, or hanging.

Therefore, it is unlikely to

fall off.

# Body and Doors – Sharp edges

Groundable

Damage

Definition

•

Sharp edge likely to cause injury or harm when contacted or grazed

Groundable

Damage

Definition

•

Sharp edge likely to cause injury or harm when contacted or grazed

Compliance Tip Compliance Tip

A sharp edge may cause injury to a pedestrian or other road user. You have the full support of Amazon Leadership in your decision on whether a sharp edge is likely to cause injury or harm when contacted of grazed.

Examples:

Examples:

Body

and

Fuel

and

Fluids

Groundable

Damage

Definition

•

Fuel cap/charging port is missing, broken or not lockable

.

•

Leaking fluid.

Fleet

Compliance

Tip

The

fuel

cap

must

be

lockable

to

prevent:

•

Fuel

being

taken

or

leaked

in

a

collision.

•

Objects

going

into

fuel

tank

e.g.

dust

and

grime.

•

Flammable

vapors

escaping

e.g.,

on

a

hot

day

there

is

an

ignition

risk

in

an

enclosed

space

when

gasoline

vapors leak out of the open fuel tank.

Body

and

Fuel

and

Fluids

Groundable

Damage

Definition

•

Fuel cap/charging port is missing, broken or not lockable

.

•

Leaking fluid.

Fleet

Compliance

Tip

The

fuel

cap

must

be

lockable

to

prevent:

•

Fuel

being

taken

or

leaked

in

a

collision.

•

Objects

going

into

fuel

tank

e.g.

dust

and

grime.

•

Flammable

vapors

escaping

e.g.,

on

a

hot

day

there

is

an

ignition

risk

in

an

enclosed

space

when

gasoline

vapors leak out of the open fuel tank.

## Example

Fuel cap is missing or broken. Ground.

The fuel cap is lockable, but the ad blue is not. Do not ground.

Fuel cover is lockable (this being covered by the plastic/metal casing). Do not ground.

The lid is broken, but the fuel cap is lockable. Do not ground.

# Body and Doors – Fuel and Fluids Continued

Fleet

Compliance

Tip

The

charging port

must

be

covered

to

prevent

d

ust, liquid and objects entering the charging port

.

T

ape

does

not provide

appropriate

adhesion

and may result in cap opening in transit

.

Missing charging port

cover.

Ground

.

Charging port cover held

on with tape.

Ground

.

Charging port cap covers

the port.

Do not ground.

Fleet

Compliance

Tip

The

charging port

must

be

covered

to

prevent

d

ust, liquid and objects entering the charging port

.

T

ape

does

not provide

appropriate

adhesion

and may result in cap opening in transit

.

Missing charging port

cover.

Ground

.

Charging port cover held

on with tape.

Ground

.

Charging port cap covers

the port.

Do not ground.

## Example

Vehicle is leaking fluid. Ground.

# Wires

Groundable

Damage

Definition

•

Loose, hanging, faulty, or frayed wire

.

Groundable

Damage

Definition

•

Loose, hanging, faulty, or frayed wire

.

## Example

Loose and hanging wires shown in the images below. Ground.

Do not ground this vehicle. No loose, hanging, faulty or frayed wires are exposed.

# Netradyne

Groundable

Damage

Definition

•

Where Netradyne is installed, external privacy sticker is not clearly visible from 6m away

(

UK 2.0

fleet only).

Groundable

Damage

Definition

•

Where Netradyne is installed, external privacy sticker is not clearly visible from 6m away

(

UK 2.0

fleet only).

You can find this sticker either in the top or bottom left corner of UK 2.0 vehicles.

## Example

Netradyne sticker.

## Example

Location of sticker on vehicle models

.

Location of sticker on vehicle models

.

# Amazon Prime decal

Example

Groundable

Damage

Definition

•

Amazon

Prime

decal

is

damaged,

missing,

or

excessively

dirty

or

any

graffiti

(

more

than

30

mm

in

diameter) on an Amazon branded

vehicl

e

.

–

Sev 2.

Example

Groundable

Damage

Definition

•

Amazon

Prime

decal

is

damaged,

missing,

or

excessively

dirty

or

any

graffiti

(

more

than

30

mm

in

diameter) on an Amazon branded

vehicl

e

.

–

Sev 2.

Graffiti more than 30mm in diameter.

Amazon Prime decal is damaged or missing on the side and rear of the vehicle.

Unapproved DSP recruitment advertisement 	Unapproved DSP logo on 2.0 vehicle.

# License Plate

Groundable

Damage

Definition

•

License plate is damaged, missing or obscured

.

Groundable

Damage

Definition

•

License plate is damaged, missing or obscured

.

## Example

License plates held on by tape. Tape does not provide appropriate Missing license plate.

support

or protection.

support

or protection.

## Example

License plate character(s) is illegible and cannot be identified by Automatic number plate recognition cameras.

## Example

License plate is on the windshield

.

License plate is on the windshield

.

# Safety Accessories

Groundable

Damage

Definition

•

Delivery

device

cradle

is

damaged,

missing,

or

is

mounted

with

a

tape,

zip

-

tie

or

similar.

Groundable

Damage

Definition

•

Delivery

device

cradle

is

damaged,

missing,

or

is

mounted

with

a

tape,

zip

-

tie

or

similar.

# Vehicle Cleanliness

Groundable

Damage

Definition

•

Interior

of

vehicle

has

excessive

grime,

dust,

or

trash

present.

•

Exterior

of

vehicle

is

visibly

dirty

when

viewed

from

6

m

away.

–

Sev 2.

•

Unapproved advertisement on a DSP 2.0 vehicle

-

Sev 2.

Groundable

Damage

Definition

•

Interior

of

vehicle

has

excessive

grime,

dust,

or

trash

present.

•

Exterior

of

vehicle

is

visibly

dirty

when

viewed

from

6

m

away.

–

Sev 2.

•

Unapproved advertisement on a DSP 2.0 vehicle

-

Sev 2.

## Example

Interior of vehicle has excessive trash present. Trash can interfere with the ability to brake and accelerate.

# Seatbelt

Groundable

Damage

Definition

•

Seatbelt or seatbelt buckle is missing, torn, frayed, damaged or not working

Groundable

Damage

Definition

•

Seatbelt or seatbelt buckle is missing, torn, frayed, damaged or not working

Fleet Compliance Tip Fleet Compliance Tip

As a guide, cuts on either edge of the webbing in excess of 2mm or in excess of 4mm away from edge, are likely to significantly reduce the seatbelt strength and therefore should be grounded. It is important to not manipulate the buckle of the seatbelt as it is a legal requirement. To note, an auditor is required to check seatbelts. Passenger side as well

Examples:

# Windshield

Groundable

Damage

Definition

•

Driver’s line of sight is obscured through dirt, stickers or devices (e.g. device cradle).

•

Windshield has crack or chips more than 10mm in driver’s line of sight (Zone A) or 40mm outside the

driver’s line of sight.

•

Wiper is missing, damaged, or not working.

•

Windshield washer system is not working and/or wiper fluid reservoir is empty

Groundable

Damage

Definition

•

Driver’s line of sight is obscured through dirt, stickers or devices (e.g. device cradle).

•

Windshield has crack or chips more than 10mm in driver’s line of sight (Zone A) or 40mm outside the

driver’s line of sight.

•

Wiper is missing, damaged, or not working.

•

Windshield washer system is not working and/or wiper fluid reservoir is empty

## Example

Line of sight diagram - Zone ‘A’. 	Outside driver’s line of sight is impacted by a crack  	than more than 40mm.

Crack

more

than

mm

10

in

driver’s

line

of

sight

).

Zone A

(

Driver’s

line

of

sight

(

Zone

A)

is

obscured

by

device

cradle.

Driver’s line of sight

zone A) is obscured.

(

Crack

more

than

mm

10

in

driver’s

line

of

sight

).

Zone A

(

Driver’s

line

of

sight

(

Zone

A)

is

obscured

by

device

cradle.

Driver’s line of sight

zone A) is obscured.

(

## Example

Wiper is missing, damaged, or not working.

# Camera

Example

Groundable

Damage

Definition

•

Back

-

up camera is loose, hanging, missing, or mounted with a tape, zip

-

tie, or similar

.

Example

Groundable

Damage

Definition

•

Back

-

up camera is loose, hanging, missing, or mounted with a tape, zip

-

tie, or similar

.

The camera is missing.

Ground.

The camera is broken.

Ground

.

Camera is attached with adhesive tape, which is not an acceptable solution

.

Ground

.

The camera is missing.

Ground.

The camera is broken.

Ground

.

Camera is attached with adhesive tape, which is not an acceptable solution

.

Ground

.

Disclaimer: All images are true examples of non-compliance witnessed during Amazon’s Vehicle Safety Audits. All non-compliant Sev1 vehicles were grounded.